# Face Masks

# Time:

- 45 mins

# Size:

- One Size Fits All

# Materials:

- Outer fabric – 15 by30 cm

- Inner fabrics – 15 by 30 cm

- 2 pieces of elastic 6mm (¼“) Wide - 20cm

- Matching or Co-Ordinating Cotton (Stitching will be seen on the side)

# Equipment:

- Pinking Shears

- Sewing machine

- Needle

# Directions:

- Cut out the pattern pieces (without the square!)

- Fold the Fabric, right sides together and cut out the 2 pattern pieces so you have 2 of each piece.

- Sew along the curves, using a ¼ inch seam allowance.

- Fold the side seams of the inside piece ¼ inch (the right side should show as you turn it over) and press.

- Sew the two parts together (right sides together) along the top and the bottom.

  - To do this you will have to make the pieces 3D and align the centre seams.

- Turn the piece right side out and press.

- Fold the side edge a tiny amount (about an ⅛”) and press.

- Fold the edge again about ½” so it aligns with the side seam of the inside piece.

  - Once sewn this will form a tube into which the elastic will be inserted.

- Stich closely to the edge of the of the tube to close the tube.

- Insert the elastic hand sew the 2 ends to form a loop.

  - For ease use tweezers to help pull the elastic through or pin a safety pin to one end to push it through.

- Twist the loop so the ends do not show by hiding them in the fabric tube.

- Insert a coffee filter into pocket or similar and wash after each use.

# Hints:

- Using a thicker elastic will make the threading of it hard work.  Don’t Do it!!!